|  | ST. VINCENT AND THE GRENADINES MARITIME ADMINISTRATION |  |
| --- | --- | --- |

CHECKLIST FOR ACCOMMODATION OF EXISTING SHIPS*

(in compliance with the provisions of Standard 3.1 of the Maritime Labour Convention, 2006)

| Name of vessel |  |
| --- | --- |
| Port of Registry | Kingstown |
| Official No. |  |
| IMO No. |  |
| Call Sign |  |
| Date of keel laid |  |

Code letters

|  |  | Yes |  | - |  | No | NA |  | Not Applicable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Ships up to 1000 gross tonnage/gross registered ton (GRT) |  |  |  |
| --- | --- | --- | --- |
|  | 1. There should be no direct openings into sleeping rooms from the spaces mentioned below. Requirement fulfilled from |  |  |
|  | 1.1 cargo spaces? |  |  |
|  | 1.2 machinery spaces? |  |  |
|  | 1.3 galleys? |  |  |
|  | 1.4 lamp rooms? |  |  |
|  | 1.5 paint rooms? |  |  |
|  | 1.6 engine, deck and other bulk store rooms? |  |  |
|  | 1.7 drying rooms, communal wash places or water closets? |  |  |
|  | 2. Are the bulkheads separating the above spaces from sleeping rooms |  |  |
|  | 2.1 constructed of steel (or other approved material)? |  |  |
|  | 2.2 watertight and gastight? |  |  |
|  | 3. Are external bulkheads of sleeping rooms and mess rooms insulated? |  |  |
|  | 4. Are machinery casings and all boundary bulkheads of galleys and spaces in which heat is |  |  |
|  | produced adjoining accommodation or passageways insulated? |  |  |
|  | 5. Have steam and/or hot water service pipes been provided with protection from heat |  |  |
|  | effect? |  |  |
|  | 6. Are internal bulkheads not likely to harbour vermin? |  |  |
|  | * Ship built before 20th August 2013. The checklist is based on the provisions of the ILO Conventions 92 and 133 as follows: - ILO C92: Art. 6.2, 6.3, 6.4, 6.5, 6.6, 6,7, 6.9, 6.11, 6.13, 7.1, 7.3, 7.4, 8.1.2.3, 10.1, 10.11, 10.12, 10.13, 10.14, 10.15, 10.17, 10.18, 10.19, 10.20, 10.24, 10.22, 10.23, 10.25, 10.26, 1.27,11.2, 11.3, 11.10, 13.2, 13.8, 13.11, 14.1, 14.2, 14.5, 14.7, 15.1, 15.2, 15.3, 15.4 & 16.3. - ILO C133: Art. 5.1, 5.2, 5.3.a, 5.3.b.i, 5.3.b.ii–iii-iv, 5.3.5, 5.3.6, 5.3.8, 5.3.10, 6.1, 6.2, 6.3, 7.1, 7.2, 7.3, 8.1, 8.2, 8.3, 8.4, 8.5, 8.6, 9.1.b, 9.2, 10, 11.1, 11.2, 11.3 & 11.4 7. Are the following rooms in the crew accommodation spaces insulated to prevent condensation or overheating |  |  |
|  | 7.1 sleeping rooms? |  |  |
|  | 7.2 mess rooms? |  |  |
|  | 7.3 recreation rooms |  |  |
|  | 7.4 alley-ways? |  |  |
|  | 8. Are main steam and exhaust pipes passing through alley-ways leading to crew |  |  |
|  | accommodations? |  |  |
|  | If yes: are they insulated and encased? |  |  |
|  | 9. Can inside panelling or sheeting be easily kept clean? |  |  |
|  | (Is tongued or grooved boarding likely to harbour vermin used) |  |  |
|  | 10. Can wall surface and deck heads in sleeping rooms and mess rooms be easily kept clean? |  |  |
|  | If painted, light in colour? |  |  |
|  | 11. Is the surface of the decks in all crew accommodations impervious to damp? |  |  |
|  | and can be easily kept clean? |  |  |
|  | 12. Has adequate drainage been provided? |  |  |
|  | 13. Are sleeping rooms and mess rooms ventilated? |  |  |
|  | 14. Has the ships if regularly engaged on voyages in the tropics and the Persian Gulf, been |  |  |
|  | provided with both mechanical means of ventilation and electric fans? (Provided that one |  |  |
|  | only of these means needs to be adopted in spaces where this ensures satisfactory |  |  |
|  | ventilation.) |  |  |
|  | 15. Has the ship, if engaged outside the tropics, been equipped with either |  |  |
|  | 15.1 mechanical means of ventilation? |  |  |
|  | 15.2 or electric fans? |  |  |
|  | 16. Has an adequate system of heating by means of steam, hot water, warm air or electricity been provided? |  |  |
|  | (not required for ships engaged exclusively in the tropics and Persian Gulf) |  |  |
|  | 17. Are the sleeping rooms situated above the load line amidships or aft? (but in no case |  |  |
|  | forward of the collision bulkhead and in passenger ships not immediately beneath working |  |  |
|  | alley-ways) |  |  |
|  | 18. Has the maximum number of persons to be accommodated in any sleeping room been |  |  |
|  | marked indelibly and legibly in the rooms where it can conveniently be seen? |  |  |
|  | 19. Has one individual berth been provided for each crew member? |  |  |
|  | 20. Berths shall not be placed side by side, fulfilled? |  |  |
|  | 21. Berths shall not be arranged in tiers more than two, fulfilled? |  |  |
|  | 22. Has the lower berths in a double tier been placed not less than 30 cm above the floor? |  |  |
|  | 23. Is the material of the framework of the berths hard, smooth and not likely to corrode or to |  |  |
|  | harbour vermin? |  |  |
|  | 24. Are tubular frames of berths completely sealed and without perforations? |  |  |
|  | 25. Has each berth been fitted with a spring bottom or a spring mattress? |  |  |
|  | and a mattress not likely to harbour vermin? |  |  |

|  | 26. Has a dust-proof material been fitted beneath the spring bottom of the upper berth if one is |  |  |
| --- | --- | --- | --- |
|  | placed over another? |  |  |
|  | 27. Has each sleeping room been provided with the following furniture of smooth, hard material not liable to warp or corrode |  |  |
|  | 27.1 clothes locker for each occupant not less than 152 cm in height and 1930 cm² cross |  |  |
|  | sectional area fitted with shelf and hasp for a padlock? |  |  |
|  | 27.2 desk or table of fixed, drop-leaf or slide-out type? |  |  |
|  | 27.3 comfortable seating accommodation? |  |  |
|  | 27.4 drawer or equivalent space for each occupant not less than 0.056 m³? |  |  |
|  | 27.5 curtains for the sidelights? |  |  |
|  | 27.6 mirror? |  |  |
|  | 27.7 small cabinets for toilet requisites? |  |  |
|  | 27.8 book rack? |  |  |
|  | 27.9 sufficient coat hooks? |  |  |
|  | 28. Have separate mess room accommodations (which have no direct neighbouring with the sleeping rooms) been provided on board ships of less than 1000 GRT for: |  |  |
|  | 28.1 master and officers? |  |  |
|  | 28.2 petty officers and other ratings? |  |  |
|  | on board ships of 1000 GRT and over for: |  |  |
|  | 28.3 master and officers? |  |  |
|  | 28.4 deck department petty officers and other ratings? |  |  |
|  | 28.5 engine department petty officers and other ratings? |  |  |
|  | 28.6 is there only one mess (with no direct neighbouring to the sleeping rooms) available on |  |  |
|  | board? |  |  |
|  | 29. Have tables and seats been made of damp-resisting material, without cracks and capable of |  |  |
|  | being easily cleaned? |  |  |
|  | 30. Have following minimum number of separate water closets been provided: |  |  |
|  | 30.1 in ships of under 800 GRT = three? |  |  |
|  | 30.2 in ships of 800-3000 GRT = four? |  |  |
|  | 30.3 in ships of 3000 GRT or over = six? |  |  |
|  | 31. Have all water closets ventilation to the open air independently of any other part of the |  |  |
|  | accommodation? |  |  |
|  | 32. Do sanitary accommodations intended for the use of more than one person comply with the following requirements: |  |  |
|  | 32.1 are floors of durable material, capable of being easily cleaned and to be properly drained |  |  |
|  | And impervious to damp? |  |  |
|  | 32.2 are the bulkheads of steel or some other equivalent material and at least watertight 23cm |  |  |
|  | above the level of the deck? |  |  |
|  | 32.3 are the accommodations sufficiently lighted, heated and ventilated? |  |  |
|  | 32.4 have water closets been situated convenient to, but separated from sleeping rooms and |  |  |
|  | washrooms without direct access from the sleeping rooms? |  |  |
|  | (except where a water closet is located in a compartment between two sleeping rooms for not more than four persons) |  |  |

|  | 33. Have separate hospital accommodations been provided in ships carrying a crew of fifteen or |  |  |
| --- | --- | --- | --- |
|  | more and engaged in a voyage of more than three days duration? |  |  |
|  | 34. Have hospital accommodations been suitable situated, such as to be easily accessible? |  |  |
|  | 35. Have water closet accommodations been provided for the exclusive use of occupants of the |  |  |
|  | hospital? |  |  |
|  | 36. Has an approved medicine chest been provided with readily understandable instructions on a |  |  |
|  | ship with no doctor? |  |  |
|  | 37. Have sufficiently ventilated accommodations convenient to the sleeping rooms been provided |  |  |
|  | for hanging oilskins outside? |  |  |
|  | 38. Has one room been provided and equipped for use as an office (applicable for ships over 3000 GRT) for: |  |  |
|  | 38.1 the deck department? |  |  |
|  | 38.2 engine department? |  |  |
|  | 39. Have provisions been made to protect the crews' quarters against mosquitoes by fitting of suitable screens to side scuttles, ventilators and doors to the open |  |  |
|  | deck? (required for ships regularly trading to mosquito-infested ports) |  |  |
|  | 40. Are ships trading regularly to or in the tropics and the Persian Gulf equipped with awnings for |  |  |
|  | use over exposed decks above crew accommodations and over recreation |  |  |
|  | deck spaces? |  |  |
|  | 41. Have separate and appropriate sleeping and living accommodations been provided on ships on |  |  |
|  | which the crew (in any department) are persons of widely different national habits and customs? |  |  |
|  | (The competent authority may modify the requirements laid down in the foregoing articles in this |  |  |
|  | case.) |  |  |
|  | REMARKS (please describe in details all items coded with '' - ''. Add separate sheet if necessary) Place and Date; Signature of MLC Inspector. |  |  |

Ships equal or greater than 1000 GRT

|  | 1. Are the floor areas per person of sleeping rooms intended for ratings not less than |  |  |
| --- | --- | --- | --- |
|  | 1.1 3.75 m² in ships of 1000-3000 GRT? |  |  |
|  | 1.2 4.25 m² in ships of 3000-10000 GRT? |  |  |
|  | 1.3 4.75 m² in ships of 10000 GRT or over? |  |  |
|  | Of sleeping rooms intended for two ratings |  |  |
|  | 1.4 2.75 m² in ships of 1000-3000 GRT? |  |  |
|  | 1.5 3.25 m² in ships of 3000-10000 GRT? |  |  |
|  | 1.6 3.75 m² in ships of 10000 GRT or over? |  |  |
|  | Of sleeping rooms intended for ratings in passenger ships |  |  |
|  | 1.7 2.35 m² in ships of 1000-3000 GRT? |  |  |
|  | 1.8 3.75 m² in ships of 3000 GRT or over? |  |  |
|  | 1.9 3.00 m² in ships of 3000 GRT or over in rooms accommodating two, three or four ² |  |  |
|  | persons? |  |  |
|  | 2. Is the number of petty officers occupying sleeping rooms not exceeding one or two persons |  |  |
|  | per room? |  |  |
|  | 3. Are the floor areas per persons of sleeping rooms for officers, where no private sitting rooms or day room is provided, not less than |  |  |
|  | 3.1 6.50 m² in ships of less than 3000 GRT? |  |  |
|  | 3.2 7.50 m² in ships of 3000 GRT or over? |  |  |
|  | 4. Have the chief navigation officer and the chief engineer in addition to their sleeping room an |  |  |
|  | adjoining sitting room or day room, where practicable in ships of 3000 GRT or over? |  |  |
|  | 5. Are the minimum inside dimensions of a berth 198 cm x 80 cm? |  |  |
|  | 6. Is the floor area of mess rooms for officers and for ratings not less than one (1) m² per person of |  |  |
|  | seating capacity? |  |  |
|  | 7. Are the mess rooms equipped with tables and seats sufficient to accommodate a greater |  |  |
|  | number of crew ²? |  |  |
|  | 8. Are there available on board |  |  |
|  | 8.1 a refrigerator of sufficient capacity for number of persons using the mess room/rooms? |  |  |
|  | 8.2 facilities for hot beverages? |  |  |
|  | 8.3 cool water facilities? |  |  |
|  | 9. Have recreation accommodations been provided for |  |  |
|  | 9.1 officers? |  |  |
|  | 9.2 ratings? |  |  |
|  | Where this is not provided separately from the mess rooms the latter should be planned, furnished and equipped to give recreational facilities. |  |  |
|  | 10. Do the furnishings for recreation accommodation include a book case and facilities for reading, |  |  |
|  | writing? |  |  |
|  | 11. Has there been provided a smoking room or library room in which films or television may be |  |  |
|  | shown and a hobby room aboard ships of 8000 GRT or over? |  |  |

|  | 12. Is at least one water closet and one tub and/or shower bath provided for every six persons |  |  |
| --- | --- | --- | --- |
|  | 12.1 for officers? |  |  |
|  | 12.2 for ratings? |  |  |
|  | 12.3 for women |  |  |
|  | 13. Are there separate private bathrooms fitted with water closet, tub and/or shower bath, wash basin with hot and cold running fresh water attached to the individual sleeping rooms |  |  |
|  | 13.1 for at least five officers in ships of 5000-15000 GRT? |  |  |
|  | 13.2 for all other officers in ships of 10000-15000 GRT private or inter-communicating |  |  |
|  | bathrooms? |  |  |
|  | 13.3 for or all officers in ships of 15000 GRT or over? (wash basin may be situated in the |  |  |
|  | sleeping room) |  |  |
|  | 14. Are there bathrooms fitted as above (item 13) for every two ratings between |  |  |
|  | adjoining sleeping rooms or opposite the entrance in ships of 25000 GRT or over? |  |  |
|  | 15. Has each sleeping room for officers or ratings been provided with a wash basin having hot and |  |  |
|  | cold running fresh water except if such wash basin is situated in a bathroom (item 13 and 14)? |  |  |
|  | 16. Are facilities (washing machines, drying machines or heated and ventilated drying rooms, irons and ironing boards or their equivalent) provided for washing, drying and ironing clothes for |  |  |
|  | 16.1 officers? |  |  |
|  | 16.2 ratings? |  |  |
|  | 17. Are there water closet and wash basin with hot and cold running fresh water on ships of 1600 GRT or over |  |  |
|  | 17.1 within easy access of the navigation bridge? |  |  |
|  | 17.2 within easy access of the machinery space if not fitted near the engine room control |  |  |
|  | centre? |  |  |
|  | 18. Are facilities for changing clothes provided on ships of 1600 GRT or over |  |  |
|  | 18.1 located outside the machinery space but with easy access to it? |  |  |
|  | 18.2 fitted with individual clothes lockers, tubs and/or shower baths and wash basins with |  |  |
|  | hot and cold running fresh water? |  |  |
|  | (If no private sleeping rooms and private or semi-private bathrooms are provided for all engine department personnel.) |  |  |
|  | 19. Is the minimum headroom in all crew accommodations not less than 198 cm? |  |  |
|  | 20. Are the crew accommodations properly lighted? |  |  |
|  | 21. Are mess rooms and sleeping rooms lighted by natural light? |  |  |
|  | and have been provided with artificial light? |  |  |
|  | 22. Has electric light been provided in crew accommodations? |  |  |
|  | as well as emergency lighting? |  |  |
|  | 23. Has an electric reading lamp been installed at the head of each berth in sleeping rooms? |  |  |

REMARKS (please describe in details all items coded with '' - ''. Add separate sheet if necessary)

| Place and Date |  |  |  | Signature of MLC Inspector |
| --- | --- | --- | --- | --- |

Space occupied by berths and lockers, chests of drawers shall be included in the measurement of the floor area.

maximum permissible number